Membrane injection system

for buildings with a basement

Article Code: MIS100ND VT63

Article number: 2900300800

GTIN: 4052487174299

Brand: Hauff-Technik

Building entry installed from the inside of the building for trenchless building connection. Can also be used for uncontrolled drilling rockets in conjunction with driving pipes OD 63 mm.

Dimensions: For core drills: ID 99 - 103 mm; For wall thicknesses: from 240 to 600 mm

Material: Front plate: glass fibre reinforced polyamide; Sealing rubber, flange, seal insert and counter-bearing: EPDM; Pipe: PVC-U; Screws and washers: stainless steel V2A (AISI 304L)

Application range: Water exposure class DIN 18533: W1-E; Waterproof concrete stress class 1 and 2

Tightness: gastight and watertight to 1.0 bar

Number of cables/medium: 1

suitable for medium pipe OD (mm): 63

Wall thickness (mm): 240 - 600

Features: no excavation work at the property; universal building entry with membrane injection system for core holes in the most common types of walls; controlled resin discharge through perforated membrane

Hauff-Technik GmbH & Co. KG

Manufacturer information

Hauff-Technik GmbH & Co. KG
Robert-Bosch-Straße 9
89568 Hermaringen, GERMANY

Telefon +49 73 22 13 33 - 0
Fax +49 73 22 13 33 - 999
office@hauff-technik.de
http://www.hauff-technik.de