Wall insert

For the installation of HSI90 system seals

Article Code: HSI90 1x1 KVB90

GTIN:  -

Brand: Hauff-Technik

Wall insert for connecting to Hateflex spiral duct 14090. Suitable for setting in concrete or for connection on the earth side. Additional HSI90 system seals can be used to seal from the wall insert at the end of the duct onto the cables.

Material: Wall insert: ABS with TPE 3-ribbed seal; Sleeve: EPDM; Clamping straps: W4; Extension pipe: PVC

Application range: Waterproof concrete stress class 1 and 2

Tightness: gastight and watertight to 2.5 bar

Features: injection molded 3-ribbed seal for pressure-tight seal to the concrete

Hauff-Technik GmbH & Co. KG

Manufacturer information

Hauff-Technik GmbH & Co. KG
Robert-Bosch-Straße 9
89568 Hermaringen, GERMANY

Telefon +49 73 22 13 33 - 0
Fax +49 73 22 13 33 - 999
office@hauff-technik.de
http://www.hauff-technik.de