Polymer spiral duct

for cable duct system

Article Code: Hauff-Flex 150/x

GTIN:  -

Brand: Hauff-Technik

The Hauff-Flex flexible polymer spiral duct and associated connection components for creating pressure-tight cable duct systems (for up to 1.0 bar external pressure). Peak compressive strength 320 N, weight 4200 g/m, minimum bending radius 900 mm, cable duct system that can be built over, empty conduit without sleeve available in lengths up to 25 m, adaptable to common cable ducts using the associated pipe connection sleeves

Dimensions: ID: 151 mm ± 1 mm; OD: 165 mm ± 1 mm

Material: Hard PVC/soft PVC

Tightness: gastight and watertight to 1.0 bar

Features: flexible polymer spiral duct (peak compressive strength 320 N); A pressure-tight cable duct system (for up to 1.0 bar external pressure) can be created using the matching connection components.

Hauff-Technik GmbH & Co. KG

Manufacturer information

Hauff-Technik GmbH & Co. KG
Robert-Bosch-Straße 9
89568 Hermaringen, GERMANY

Telefon +49 73 22 13 33 - 0
Fax +49 73 22 13 33 - 999
office@hauff-technik.de
http://www.hauff-technik.de